| No | Komponen | Pin Komponen | Terhubung ke (ESP32) | Keterangan |
| --- | --- | --- | --- | --- |
| 1 | DHT22 #1 | VCC | 3.3V | Catu daya sensor |
|  |  | GND | GND | Ground |
|  |  | DATA | GPIO 4 | Tambah resistor pull-up 4.7k ke 3.3V |
| 2 | DHT22 #2 | VCC | 3.3V | Catu daya |
|  |  | GND | GND | Ground |
|  |  | DATA | GPIO 16 | Pull-up 4.7k ke 3.3V |
| 3 | LDR #1 | Output pembagi | GPIO 32 (ADC1) | 100k ke GND |
| 4 | LDR #2 | Output pembagi | GPIO 33 (ADC1) | 100k ke GND |
| 5 | LDR #3 | Output pembagi | GPIO 34 (ADC1) | Input-only |
| 6 | LDR #4 | Output pembagi | GPIO 35 (ADC1) | Input-only |
| 7 | LDR #5 | Output pembagi | GPIO 36 (ADC1) | Input-only (VP) |
| 8 | LDR #6 | Output pembagi | GPIO 39 (ADC1) | Input-only (VN) |
| 9 | Proximity 1 | VCC | 3.3V / 5V | Sesuai modul |
|  |  | GND | GND | Ground |
|  |  | OUT | GPIO 13 | Digital input |
| 10 | Proximity 2 | VCC | 3.3V / 5V | Sesuai modul |
|  |  | GND | GND | Ground |
|  |  | OUT | GPIO 14 | Digital input |
| 11 | IR Receiver (TSOP1838) | VCC | 3.3V | Power |
|  |  | GND | GND | Ground |
|  |  | OUT | GPIO 15 | Capture sinyal remote AC |
| 12 | IR Transmitter (IR LED + Transistor) | IR LED Anoda | 3.3V → resistor 100Ω | Pembatas arus |
|  |  | IR LED Katoda | Kolektor transistor | Output IR |
|  |  | Transistor Base | GPIO 17 lewat resistor 1k | Driver penguat sinyal IR |
|  |  | Transistor Emitter | GND | Ground |
| 13 | TFT LCD SPI (ILI9341) | SCK | GPIO 18 | Clock SPI |
|  |  | MOSI | GPIO 23 | Data ke LCD |
|  |  | MISO | GPIO 19 | Data ke ESP32 |
|  |  | CS | GPIO 5 | Chip select |
|  |  | DC | GPIO 21 | Data/command |
|  |  | RST | GPIO 22 | Reset LCD |
|  |  | VCC | 5V / 3.3V | Sesuai modul LCD |
|  |  | GND | GND | Ground |
| 14 | Relay 4 Channel | VCC | 5V | Catu daya relay |
|  |  | GND | GND ESP32 | Ground |
|  |  | IN1 | GPIO 25 | Relay channel 1 |
|  |  | IN2 | GPIO 26 | Relay channel 2 |
|  |  | IN3 | GPIO 27 | Relay channel 3 |
|  |  | IN4 | GPIO 12 | Relay channel 4 (aman output) |
| 15 | Power Supply | Input | 220V AC | Sumber utama |
|  |  | Output | 5V untuk relay, 3.3V untuk ESP32 | Pastikan arus cukup |
|  |  | Proteksi | MCB / Sekering | Pengaman sistem |